Experiment No: 01 [Example 2.1(a)]

Name of Experiment: Generate and Plot xn=2n+2-n-4,   -5≤n≤5

| Program Code: function [x,n] = impseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) == 0]; clc; clear; close all; n = [-5:5]; x = 2*impseq(-2,-5,5) - impseq(4,-5,5); stem(n,x); title('Sequence in Problem 2.1a'); xlabel('n'); ylabel('x(n)'); | Output: |
| --- | --- |

Experiment No: 02 [Example 2.1(b)]

Name of Experiment: Generate and Plot

xn=nun-un-10+10e-0.3n-10un-10-un-20   0≤n≤20

| Program Code: function [x,n] = stepseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) >= 0]; n = [0:20]; x1 = n.*(stepseq(0,0,20)-stepseq(10,0,20)); x2 = 10*exp(-0.3*(n-10)).*(stepseq(10,0,20)-stepseq(20,0,20)); x = x1+x2; subplot(2,2,3); stem(n,x); title('Sequence in Problem 2.1b'); xlabel('n'); ylabel('x(n)'); | Output: |
| --- | --- |

Experiment No: 03 [Example 2.1(c)]

Name of Experiment: Generate and Plot

xn=cos0.04πn+0.2ωn;                        0≤n≤50

| Program Code: n = [0:50]; x = cos(0.04*pi*n)+0.2*randn(size(n)); subplot(2,2,2); stem(n,x); title('Sequence in Problem 2.1c'); xlabel('n'); ylabel('x(n)'); | Output: |
| --- | --- |

Experiment No: 04 [Example 2.1(d)]

Name of Experiment: Generate and Plot

xn=…..,5,4,3,2,1,5,4,3,2,1,5,4,3,2,1,……..;         -10≤n≤10

| Program Code: n = [-10:9]; x = [5,4,3,2,1]; xtilde = x' * ones(1,4); xtilde = (xtilde(:))'; subplot(2,2,4); stem(n,xtilde); title('Sequence in Problem 2.1d'); xlabel('n'); ylabel('xtilde(n)'); | Output: |
| --- | --- |

Experiment No: 05 [Example 2.2 (a, b)]

Name of Experiment: Determine and Plot, Let xn={1,2,3,4,5,6,7,6,5,4,3,2,1}

x1n=2xn-5-3xn+4

x2n=x3-n+xnx(n-2)

| Program Code 2.2(a): function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; function [y,n] = sigshift(x,m,k) n = m+k; y = x; n = -2:10; x = [1:7,6:-1:1]; [x11,n11] = sigshift(x,n,5); [x12,n12] = sigshift(x,n,-4); [x1,n1] = sigadd(2*x11,n11,-3*x12,n12); subplot(2,1,1); stem(n1,x1); title('Sequence in Example 2.2a'); xlabel('n'); ylabel('x1(n)'); | Program Code 2.2(b): function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; function [y,n] = sigshift(x,m,k) n = m+k; y = x; function [y,n] = sigmult(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1 .* y2; function [y,n] = sigfold(x,n) y = fliplr(x); n = -fliplr(n); [x21,n21] = sigfold(x,n); [x21,n21] = sigshift(x21,n21,3); [x22,n22] = sigshift(x,n,2); [x22,n22] = sigmult(x,n,x22,n22); [x2,n2] = sigadd(x21,n21,x22,n22); subplot(2,1,2); stem(n2,x2); title('Sequence in Example 2.2b'); xlabel('n'); ylabel('x2(n)'); |
| --- | --- |

Output:

Experiment No: 06 [Example 2.3]

Name of Experiment: Generate the complex-valued signal

xn=e-0.1+j0.3n               -10≤n≤10

| Program Code: clc; clear; close all; n = [-10:1:10]; alpha = -0.1+0.3j; x = exp(alpha*n); subplot(2,2,1); stem(n,real(x)); title('real part'); xlabel('n'); subplot(2,2,2); stem(n,imag(x)); title('imaginary part'); xlabel('n'); subplot(2,2,3); stem(n,abs(x)); title('magnitude part'); xlabel('n'); subplot(2,2,4); stem(n,(180/pi)*angle(x)); title('phase part'); xlabel('n'); | Output: |
| --- | --- |

Experiment No: 07 [Example 2.4]

Name of Experiment: Let x(n) = u(n) - u(n - 10). Decompose c(n) into even and odd components.

| Program Code: function [x,n] = stepseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) >= 0]; function [xe, xo, m] = evenodd(x,n) if any(imag(x) ~= 0) error('x is not a real sequence') end m = -fliplr(n); m1 = min([m,n]); m2 = max([m,n]); m = m1:m2; nm = n(1)-m(1); n1 = 1:length(n); x1 = zeros(1,length(m)); x1(n1+nm) = x; x = x1; xe = 0.5*(x + fliplr(x)); xo = 0.5*(x - fliplr(x)); n = [0:10]; x = stepseq(0,0,10)-stepseq(10,0,10); [xe,xo,m] = evenodd(x,n); subplot(2,2,1); stem(n,x); title('Rectangular pulse'); xlabel('n'); ylabel('x(n)'); axis([-10,10,0,1.2]); subplot(2,2,2); stem(m,xe); title('Even Part'); xlabel('n'); ylabel('xe(n)'); axis([-10,10,0,1.2]); subplot(2,2,4); stem(m,xo); title('Odd Part'); xlabel('n'); ylabel('xe(n)'); axis([-10,10,-0.6,0.6]); | Output: |
| --- | --- |

Experiment No: 08 [Example 2.5]

Name of Experiment: Let the rectangular pulse x(n) = u(n) - u(n - 10) of Example 2.4 be an input to an LTI system with impulse response hn=0.9nun Determine y(n).

| Program Code: function [x,n] = stepseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) >= 0]; n = -5:50; u1 = stepseq(0,-5,50); u2=stepseq(10,-5,50); x = u1-u2; h = ((0.9).^n).*u1; subplot(1,1,1) subplot(2,1,1); stem(n,x); axis([-5,50,0,2]) title('Input Sequence') xlabel('n'), ylabel('x(n)') subplot(2,1,2); stem(n,h); axis([-5,50,0,2]) | title('Impulse Response') xlabel('n'), ylabel('h(n)'); pause print -deps2 ex0205a.eps % output response y = (10*(1-(0.9).^(n+1))).*(u1-u2)+(10*(1-(0.9)^10)*(0.9).^(n-9)).*u2; subplot(1,1,1) subplot(2,1,2); stem(n,y); axis([-5,50,0,8]) title('Output Sequence') xlabel('n'), ylabel('y(n)') print -deps2 ex0205b.eps |
| --- | --- |

Output:

Experiment No: 09 [Example 2.6]

Name of Experiment: Give the following two sequences

xn=3,11,7,0,-1,4,2       -3≤n≤3

and hn=2,3,0,-5,2,1             -1≤n≤4

| Program Code: x = [3,11,7,0,-1,4,2]; h = [2,3,0,-5,2,1]; y = conv(x,h); | Output: |
| --- | --- |

Experiment No: 10 [Example 2.7]

Name of Experiment: Perform the convolution in Example 2.6 using the conv_m function.

| Program Code: x = [3,11,7,0,-1,4,2]; nx = [-3:3]; h = [2,3,0,-5,2,1]; ny = [-1:4]; [y,ny] = conv_m(x,nx,h,ny); | Output: |
| --- | --- |

Experiment No: 11 [Example 2.8]

Name of Experiment: Cross correlation sequence. Let

xn=[3,11,7,0,-1,4,2]

yn=xn-2+ωn

| Program Code: function [y,ny] = conv_m(x,nx,h,nh) nyb = nx(1)+nh(1); nye = nx(length(x)) + nh(length(h)); ny = [nyb:nye]; y = conv(x,h); function [y,n] = sigfold(x,n) y = fliplr(x); n = -fliplr(n); function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; function [y,n] = sigshift(x,m,k) n = m+k; y = x; %noise sequence 1 x = [3,11,7,0,-1,4,2]; nx=[-3:3]; [y,ny] = sigshift(x,nx,2); w=randn(l,length(y)); nw=ny; [y,ny] = sigadd(y,ny,w,nw); [x,nx] = sigfold(x,nx); [rxy,nrxyl = conv_m(y,ny,x,nx); subplot(1,1,1), subplot(2,1,1); stem(nrxy,rxy); axis([-5,10,-50,25O]); xlabel('lag variable 1') ylabel('rxy'); title('Crosscorrelation: noise sequence 1'); | %noise sequence 2 x = [3, 11, 7, 0, -1, 4, 2]; nx = [-3:3]; [y,nyl= sigshift(x,nx,2); w = randn(1,length(y)); nw = ny; [y,ny] = sigadd(y,ny,w,nw); [x,nx] = sigfold(x,nx); [rxy,nrxy] = conv_m(y,ny,x,nx); subplot(2,1,2); stem(nrxy,rxy); axis([-5,10,-50,250); xlabel('lag variable 1'); ylabel('rxy'); title('Crosscorrelation: noise sequence 2'); |
| --- | --- |

Output:

Experiment No: 12 [Example 2.9 (a, b)]

Name of Experiment: Given the following difference equation

yn-yn-1+0.9yn-2=xn

- Calculate and plot the impulse response hn at n=-20,………, 100.

- Calculate and Plot the unit step sequence response δn at n=-20,………, 100.

| Program Code 2.9(a): function [x,n] = impseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) == 0]; b = [1]; a = [1,-1,0.9]; x = impseq(0,-20,120); n = [-20:120]; h = filter(b,a,x); subplot(2,1,1); stem(n,h); title('Impulse Response'); xlabel('n'); ylabel('h(n)'); | Program Code 2.9(b): function [x,n] = stepseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) >= 0]; x = stepseq(0,-20,120); s = filter(b,a,x); subplot(2,1,2); stem(n,s); title('Step Response'); xlabel('n'); ylabel('s(n)'); |
| --- | --- |

Output:

Experiment No: 13 [Example 2.10]

Name of Experiment: Let us consider the convolution given in Example 2.5. The input sequence is of finite duration. xn=un-u(n-10). While the impulse response is of infinite duration. hn=0.9nu(n)

Determine yn=xn*h(n)

| Program Code: function [x,n] = stepseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) >= 0]; b = [1]; a = [1,-0.9]; n = -5:50; x = stepseq(0,-5,50) - stepseq(10,-5,50); y = filter(b,a,x); subplot(1,1,1); subplot(2,1,2); | stem(n,y); title('Output Sequence'); xlabel('n'); ylabel('y(n)'); axis([-5,50,-0.5,8]); |
| --- | --- |

Output:

Experiment No: 14

Name of Experiment: Linear Convolution.

| Program Code: function[y] = linearconvulation(x,h) n1 = length(x); n2 = length(h); N = (n1+n2) - 1; x1 = [x zeros(1, N-n1)]; h1 = [h zeros(1, N-n2)]; for n = 1:N for m = 1:N if n>m H(m,n) = 0; else H(m,n) = h1(m - (n-1)); end end end y = H * x1'; end | clc; clear; close all; x = [3,11,7,0,-1,4,2]; h = [2,3,0,-5,2,1]; [linearoutput] = linearconvulation(x,h); figure(1); plot(linearoutput); |
| --- | --- |

Output:

Experiment No: 15

Name of Experiment: Circular Convolution.

| Program Code: function[z] = circularconvulation(x1,x2) n1 = length(x1); n2 = length(x2); N = max(n1,n2); x1 = [x1 zeros(1, N-n1)]; x2 = [x2 zeros(1, N-n2)]; s = n1 - n2; if(s==0) x2 = [x2 zeros(1,s)]; else x1 = [x1 zeros(1,-s)]; x2 = [x2 zeros(1,s)]; end %circular multiplication z = []; for k = 1:N y = 0; for i = 1:N j = (k-i) + 1; if(j<=0) j = j + N; end y = y + (x1(i) * x2(j)); end z = [z,y]; end end | clc; clear; close all; x = [3,11,7,0,-1,4,2]; h = [2,3,0,-5,2,1]; [circularoutput] = circularconvulation(x,h); figure(1); plot(circularoutput); |
| --- | --- |

Output:

Experiment No: 16 [Problem 2.1(a)]

Name of Experiment: x1n= m=010m+1δn-2m-δn-2m-1; 0≤n≤25

| Program Code: function [x,n] = impseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) == 0]; clc; clear; close all; n1 = [0:25]; x1 = zeros(1,length(n1)); for m = 0:10 x1 = x1 + (m+1)*(impseq(2*m,0,25) - impseq(2*m+1,0,25)); end stem(n1,x1); xlabel('n'); ylabel('x1(n)'); title('Sequence x1(n)'); | Output: |
| --- | --- |

Experiment No: 17 [Problem 2.1(b)]

Name of Experiment:

x2n= n2un+5-nn-6+10δn+200.5n[un-4-un-10]

| Program Code: function [x,n] = impseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) == 0]; function [x,n] = stepseq(n0,n1,n2) n = [n1:n2]; x = [(n-n0) >= 0]; clc; clear; close all; n2 = -5:10; x2 = (n2.^2).*(stepseq(-5,-5,10)-stepseq(6,-5,10))+10*impseq(0,-5,10)+20*((0.5).^n2).*(stepseq(4,-5,10)-stepseq(10,-5,10)); stem(n2,x2); xlabel('n'); ylabel('x1(n)'); title('Sequence x2(n)'); | Output: |
| --- | --- |

Experiment No: 18 [Problem 2.1(c)]

Name of Experiment:

x3n=0.9ncos0.2πn+π3           0≤n≤20

| Program Code: clc; clear; close all; n3 = [0:20]; x3 = ((0.9).^n3).*cos(0.2*pi*n3+pi/3); stem(n3,x3); xlabel('n'); ylabel('x3(n)'); title('Sequence x3(n)'); | Output: |
| --- | --- |

Experiment No: 19 [Problem 2.1(d)]

Name of Experiment:

x4n=10cos0.0008πn2+ω(n)           0≤n≤100

| Program Code: clc; clear; close all; w = 2*(rand(1,101)-0.5); n4 = [0:100]; x4 = 10*cos(0.0008*pi*n4.^2)+w; stem(n4,x4); xlabel('n'); ylabel('x4(n)'); title('Sequence x4(n)'); | Output: |
| --- | --- |

Experiment No: 20 [Problem 2.1(e)]

Name of Experiment:

x5n=……,1,2,3,2,1,2,3,2,1,…….      Plot 5 Periods

| Program Code: clc; clear; close all; n5 = [-8:11]; x5 = [2,1,2,3]; x5 = x5'*ones(1,5); x5 = (x5(:))'; stem(n5,x5); xlabel('n'); ylabel('x5(n)'); title('Sequence x5(n)'); | Output: |
| --- | --- |

Experiment No: 21 [Problem 2.2(a)]

Name of Experiment: Let xn={1,-2,4,6,-5,8,10}

x1n=3xn+2+xn-4-2x(n)

| Program Code: function [y,n] = sigshift(x,m,k) n = m+k; y = x; function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; n = [-4:2]; x = [1,-2,4,6,-5,8,10]; [x11,n11] = sigshift(3*x,n,-2); [x12,n12] = sigshift(x,n,4); [x13,n13] = sigadd(x11,n11,x12,n12); [x1,n1] = sigadd(x13,n13,2*x,n); stem(n1,x1); xlabel('n'); ylabel('x1(n)'); title('Sequence x1(n)'); | Output: |
| --- | --- |

Experiment No: 22 [Problem 2.2(b)]

Name of Experiment: Let xn={1,-2,4,6,-5,8,10}

x2n=5x5+n+4xn+4+3x(n)

| Program Code: function [y,n] = sigshift(x,m,k) n = m+k; y = x; function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; n = [-4:2]; x = [1,-2,4,6,-5,8,10]; [x21,n21] = sigshift(5*x,n,-5); [x22,n22] = sigshift(4*x,n,-4); [x23,n23] = sigadd(x21,n21,x22,n22); [x2,n2] = sigadd(x23,n23,3*x,n); stem(n2,x2); xlabel('n'); ylabel('x2(n)'); title('Sequence x2(n)'); | Output: |
| --- | --- |

Experiment No: 23 [Problem 2.2(c)]

Name of Experiment: Let xn={1,-2,4,6,-5,8,10}

x3n=xn+4xn-1+x2-nx(n)

| Program Code: function [y,n] = sigshift(x,m,k) n = m+k; y = x; function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; function [y,n] = sigmult(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1 .* y2; function [y,n] = sigfold(x,n) y = fliplr(x); n = -fliplr(n); | n = [-4:2]; x = [1,-2,4,6,-5,8,10]; [x31,n31] = sigshift(x,n,-4); [x32,n32] = sigshift(x,n,1); [x33,n33] = sigmult(x31,n31,x32,n32); [x34,n34] = sigfold(x,n); [x34,n34] = sigshift(x34,n34,2); [x34,n34] = sigmult(x34,n34,x,n); [x3,n3] = sigadd(x33,n33,x34,n34); stem(n3,x3); xlabel('n'); ylabel('x3(n)'); title('Sequence x3(n)'); |
| --- | --- |

Output:

Experiment No: 24 [Problem 2.2(d)]

Name of Experiment: Let xn={1,-2,4,6,-5,8,10}

x4n=2e0.5nxn+cos0.1πnxn+2,   -10≤n≤10

| Program Code: function [y,n] = sigshift(x,m,k) n = m+k; y = x; function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; function [y,n] = sigmult(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1 .* y2; n4 = [-10:10]; n = [-4:2]; x = [1,-2,4,6,-5,8,10]; x41 = 2*exp(0.5*n4); x412 = cos(0.1*pi*n4); [x42,n42] = sigmult(x41,n4,x,n); [x43,n43] = sigshift(x,n,-2); [x44,n44] = sigmult(x412,n4,x43,n43); [x4,n4] = sigadd(x42,n42,x44,n44); stem(n4,x4); xlabel('n'); ylabel('x4(n)'); title('Sequence x4(n)'); | Output: |
| --- | --- |

Experiment No: 25 [Problem 2.2(e)]

Name of Experiment: Let xn={1,-2,4,6,-5,8,10}

x5n=k=15nx(n-k)

| Program Code: function [y,n] = sigshift(x,m,k) n = m+k; y = x; function [y,n] = sigadd(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1+y2; function [y,n] = sigmult(x1,n1,x2,n2) n = min(min(n1),min(n2)):max(max(n1),max(n2)); y1 = zeros(1,length(n)); y2 = y1; y1(find((n>=min(n1))&(n<=max(n1))==1))=x1; y2(find((n>=min(n2))&(n<=max(n2))==1))=x2; y = y1 .* y2; | n = [-4:2]; x = [1,-2,4,6,-5,8,10]; [x51,n51] = sigshift(x,n,1); [x52,n52] = sigshift(x,n,2); [x5,n5] = sigadd(x51,n51,x52,n52); [x53,n53] = sigshift(x,n,3); [x5,n5] = sigadd(x5,n5,x53,n53); [x54,n54] = sigshift(x,n,4); [x5,n5] = sigadd(x5,n5,x54,n54); [x55,n55] = sigshift(x,n,5); [x5,n5] = sigadd(x5,n5,x55,n55); [x5,n5] = sigmult(x5,n5,n5,n5); stem(n5,x5); xlabel('n'); ylabel('x5(n)'); title('Sequence x5(n)'); |
| --- | --- |

Output: